Product Safety Data Sheet

1. Product And Company Identification

Product:			Charm Furniture Polish, 300ml - All variants

Manufacturer:		Sanmex International Ltd			Phone Number:	00+44 (0)141 647 2244

Dalmarnock Road				Fax Number:	00+44 (0)141 613 1228

Rutherglen				e mail		info@sanmex.com

Glasgow G73 1NY

United Kingdom

2. Composition/Information On Components

| Contains: | EINECS Number | CAS Number | % Wt/Wt |  | Classification |
| --- | --- | --- | --- | --- | --- |
| White Spirit | 265-185-4 | 64742-82-1 | 16.00 |  | Xn, N; R10, R51/53, R65, R66 |
| n-Butane, Isobutane, Propane mixture | 270-704-2 | 68476-85-7 | 12.50 |  | F+; R12 |
| Sodium Nitrite | 231-555-9 | 007632-00-0 | 0.3 |  | O, T, N; R8, R25, R50 |

3. Hazards Identification

F+ Extremely Flammable. Pressurised Container.

4. First-Aid Measures

Eyes:		Rinse with water for 15 minutes.

Skin:		Wash with soap and water.

Inhalation:	Remove the person to fresh air. If breathing has stopped administer artificial respiration. Obtain medical attention.

Ingestion:	Wash out mouth with water. Do not induce vomiting. Give water to drink. Seek medical attention.

5. Fire-Fighting Measures

Use foam, carbon dioxide or dry powder extinguishers.

6. Accidental Release Measures

Absorb with any inert material. Ensure adequate ventilation. Remove sources of ignition.

7. Handling and Storage

Keep away from sources of ignition. Do not smoke while using. Keep at temperature not exceeding 50°C. Protect from sunlight. Keep out of the reach of children. Intentional misuse by deliberately inhaling the contents can be harmful or fatal.

8. Exposure Controls/Personal Protection

Use only as directed on the can. Avoid contact with the skin and eyes.

9. Physical and Chemical Properties

Appearance:	Pale yellow Liquid

Odour:		As fragrance

pH:		6.0 - 8.0

10. Stability and Reactivity

As with all aerosol cans, protect from sunlight and do not expose to temperatures exceeding 50°C.

11. Toxicological Information

Eyes:		May cause irritation.

Skin:		May irritate sensitive skin.

Ingestion:	May cause nausea and vomiting.

Inhalation:	May cause dizziness, stupor or unconsciousness depending on the extent of exposure.

12. Ecological Information

Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

13. Disposal Considerations

Do not pierce or burn even after use. Dispose according to national and local authority guidelines.

14. Transport Information

| Transport Type | ADR (Road/Rail) | SEA | AIR |
| --- | --- | --- | --- |
| UN Number | UN 1950 | UN 1950 | UN 1950 |
| Class | 2.1 | 2.1 | 2.1 |
| Proper shipping name | Aerosols | Aerosols | Aerosols, flammable |
| Packing group | None | None | None |
| Marine pollutant | N/A | No | N/A |
| Other information | ‘Limited quantity’ provisions apply. No Tremcard required. | ‘Limited quantity’ provisions apply. | ‘Limited quantity’ provisions apply. Hazard label required: Flammable Gas. |

15. Regulatory Information

| Symbol: |  |
| --- | --- |

Risk Phrases:		R-12, Extremely Flammable

R 52/53, Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic 			environment.

Safety Phrases:		S-2, Keep out of the reach of children. S-16, Keep away from sources of ignition - NO 				SMOKING. S-23, Do not breathe spray, 	S- 47. Keep at temperature not exceeding 50°C.

16. Other information

R phrases used in this document

| R8 Contact with combustible material may cause fire | R51/53 Toxic to aquatic organisms, may cause long-term |
| --- | --- |
| R10 Flammable | adverse effects in the aquatic environment. |
| R12 Extremely Flammable | R65 Harmful: may cause lung damage if swallowed. |
| R25 Toxic if swallowed | R66 Repeated exposure may cause skin dryness or cracking. |
| R50 Very toxic to aquatic organisms |  |

This Safety Data does not constitute the users own assessment of workplace risk.